| AHD Ltd |  |
| --- | --- |
| 1229 Maraekakaho Road Hastings 4175 New Zealand (06) 873 3611 | Emergency phone number: 0800 734 607 (24 hours) National Poisons Centre: 0800 764 766 (0800 POISON) Or CHEMCALL 0800-243-622 The Emergency Services: Phone 111 |

HAZNOTE                                 Phone number: 027 432 3953			Date: 06 December 2024

| Product name: Unlock Pour on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EMERGENCY INFORMATION & STORAGE GUIDE & DANGEROUS GOODS DECLARATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substance Identification: Liquid containing…Diflubenzuron (CAS No. 35367-38-5), Deltamethrin (CAS No. 52918-63-5), Formalin (CAS No. 50-00-0). Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (contains Diflubenzuron, Deltamethrin) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UN NO. 3082 | DG Class 9 |  |  |  | Subsidiary Risk class | Packing Group III | Flash Point >1000C |  |  | HAZCHEM 3Z |  | Marine Pollutant Yes |  |  | Maximum Transport Quantity 1000 L |
| GHS Classifications Eye irritation Category 2 Skin sensitisation Category 1 Specific target organ toxicity (repeated exposure) Category 2 Hazardous to the aquatic environment acute Category 1 Hazardous to the aquatic environment chronic Category 1 |  |  |  |  | HSNO Approval No HSR100759 |  | Tracking required No |  |  | Approved Handler No |  |  |  |  | IER Guide Not available |
| EMERGENCY PROCEDURES: Emergency phone number: 0800 734 607 (24 hours) National Poisons Centre: 0800 764 766 (0800 POISON). The Emergency Services: Phone 111 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IF THIS HAPPENS |  |  | DO THIS |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle accident |  |  | Call the emergency services on 111. |  |  |  |  |  |  |  |  |  |  |  |  |
| Spill or leak |  |  | Small spills/leaks: Clean up and absorb spills with inert material (sand, soil, hydrated lime or vermiculite) or another absorbent material (i.e. paper towel). and place in waste containers, labelled for disposal. Contain in a secure location until disposal method is established. Dispose of waste safely. Avoid release to the environment. Large spills/leaks: Prevent spillage from entering drains or water ways and call emergency services. |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire |  |  | Non flammable. Use extinguishing media suited to burning materials (i.e. Carbon dioxide, Dry chemical, Foam, Water fog) In the event of fire, wear self-contained breathing apparatus. Use personal protective equipment. |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If swallowed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If in eyes |  | Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If on skin |  | Wash skin well with plenty water. Get medical attention if irritation persists. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If inhaled |  | If inhaled, Remove to fresh air. Get medical attention if symptoms persist. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STORAGE AND SEGREGATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Storage Store locked up in the original container, tightly closed in a safe place. Store below 300C away from direct sunlight. Keep out of reach of children. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aggregate Storage Volume Thresholds: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location certificate NA | Hazardous Atmosphere Zone NA |  |  | Number of fire extinguishers NA |  | Hazard Class & Emergency Signage ≥100 L |  |  | Emergency Information Any quantity |  | Emergency Plan ≥100 L |  |  | Secondary containment ≥100 L |  |
| Do not store or load with: No special requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Segregate from: N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DANGEROUS GOODS DECLARATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number and type of packages: |  |  |  |  |  |  |  | Gross weight: kg |  |  |  |  | Volume: |  |  |
| Consignor: |  |  |  |  |  |  |  | Consignee: |  |  |  |  |  |  |  |
| Dangerous Goods Declaration: I hereby declare that the contents of the consignment are accurately described above by the proper shipping name and are classified, packaged, and identified according to the national rules, regulations, and legislation applicable to the transport of this substance. Name: Title: Signature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |